TCM 264-LAN Session B

Lab 7-Troubleshooting

Aaron Hurt

Steve Klesper

Tom Ondick

Shane Bridger

Aimee Black

Sean Jenkins

Mike Freeman

- Lab purpose:

  - To diagnose and correct any and all issues preventing machines in row G from connecting to the network.

- General issues and solutions:

  - No link lights to the switch from the hub

    - Solution:	Press down link button on hub

  - Incorrect cabling on wall plate

    - Solution: Swap cables in correct order

  - Bad hub port number 6 connected to machine G06

    - Solution: Swap G06 to hub port number 7

- Machine specific issues and solutions:

  - TCP/IP not installed or configured on G02

    - Solution: Reinstall TCP/IP and configure for DHCP

  - Incorrect monitor output on G04 or G05

    - Solution: Switch monitor cables on G04 and G05

  - Bad hard drive on G06

    - Solution: Replace hard drive with a good drive from the crib

- Final outcome:

  - Able to ping external IP addresses and connect to Devry network (Novell side) from all machines on row (Macintosh machine not tested)